Graphical Processing Units

| GPU Model | Architecture | VRAM | Memory Bandwidth | FP16 TFLOPS | FP8 TFLOPS |
| --- | --- | --- | --- | --- | --- |
| Blackwell B300 (Ultra) | Blackwell Ultra | 288 GB HBM3e | ~8 TB/s | ~2,500 TFLOPS | ~4,500 TFLOPS |
| Blackwell B200 | Blackwell | 192 GB HBM3e | ~8 TB/s | ~2,500 TFLOPS | ~4,500 TFLOPS |
| H200 | Hopper | 141 GB HBM3e | ~4.8 TB/s | ~1,979 TFLOPS | ~3,958 TFLOPS |
| H100 (80 GB) | Hopper | 80 GB HBM3 | ~3.35 TB/s | ~1,979 TFLOPS | ~3,958 TFLOPS |
| A100 (80 GB) | Ampere | 80 GB HBM2e | ~2 TB/s | ~624 TFLOPS | No native FP8 |
| L40S (or L40) | Ada Lovelace | 48 GB GDDR6 | ~864 GB/s | ~362–724 TFLOPS | ~724 TFLOPS |
| L4 | Ada Lovelace | 24 GB GDDR6 | ~300 GB/s | ~242 TFLOPS | ~485 TFLOPS |
| RTX 5090 | Blackwell | 32 GB GDDR7 | ~1792 GB/s | ~105 TFLOPS | ~220 TFLOPS |
| RTX 4090 | Ada Lovelace | 24 GB GDDR6X | ~1 TB/s | ~82.58 TFLOPS | 165 TFLOPS |
| T4 (for reference) | Turing | 16 GB GDDR6 | ~320 GB/s | Low (~65 TFLOPS) | Low |

| GPU Model | Notes |
| --- | --- |
| Blackwell B300 (Ultra) | Top AI compute & largest memory; massive AI training/inference; next-gen Blackwell |
| Blackwell B200 | Best next-gen AI performance before Ultra; huge throughput for LLMs. |
| H200 | Big memory + very high FP16/FP8 for large model training/inference. |
| H100 (80 GB) | Prev-gen AI workhorse, still extremely strong. |
| A100 (80 GB) | Classic AI/HPC chip; lower FP16 compared to Hopper. |
| L40S (or L40) | Mid-range enterprise GPU for inference & mixed workloads. |
| L4 | Efficient inference / small-model workloads. |
| RTX 5090 | RAM process is better than L4, but model training speed is less than that in L4. Mainly used for the gaming. |
| RTX 4090 | Top consumer GPU with FP16/FP8 support but not optimized like data-center cards. |
| T4 (for reference) | Legacy inference-focused GPU (entry). |